# Research Paper Review & Assignment 2 Comparison

COMP/ENGG — Multimedia Compression & Streaming
Assignment 2: DASH Adaptive Bitrate Streaming Client

# Table of Contents

1.  Paper Summaries

1.1  VV-DASH (MMSys 2025)

1.2  Low Latency Live Streaming in DASH and HLS (MM 2022)

1.3  DASH Fast Start using HTTP/2 (NOSSDAV 2015)

2.  Comparison Table

3.  Five Technical Gaps in Assignment 2

4.  What Must Be Done to Raise the Deliverables

5.  Five Common Issues & How to Deal with Them

# 1. Paper Summaries

## 1.1  VV-DASH: A Framework for Volumetric Video DASH Streaming

Heidarirad & Wang. ACM MMSys 2025. https://doi.org/10.1145/3712676.3718339

### Problem Statement

Conventional DASH frameworks cannot handle 3D point-cloud video. Draco encodes frame-by-frame, requiring 30 HTTP requests/second at 30 fps — impractical overhead. V-PCC uses a proprietary format incompatible with standard DASH players. No codec-agnostic container existed to package volumetric video into DASH-ready segments.

### Proposed Solution

DVV binary segment format — version, header encoding, timescale, codec ID (DRCO/VPCC), plus offset/size/PTS per payload.

Parallel decoding — 30 Draco frames batched into one 1-second DVV segment; frames decoded in parallel at the client.

V-PCC integration — each V3C bitstream (2 frames) = one DVV payload; TMC2-RS decoder (15× faster than reference).

Testbed — NGINX + HAProxy (Linux TC) + iStream player, fully containerised.

### Attained Results

Decoding time reduced by up to 51 % through batch parallelism.

Average streaming bitrate improved by ~37.8 % over frame-by-frame Draco.

13.2 % system throughput gain (segment turnaround: 0.91 s vs. >1 s for 30 frames).

Full DASH sessions demonstrated for both Draco and V-PCC with smooth and stall-recovery profiles.

## 1.2  Low Latency Live Streaming Implementation in DASH and HLS

Bentaleb et al. ACM MM 2022. https://doi.org/10.1145/3503161.3548544

### Problem Statement

Four LL-DASH ABR algorithms (LoL+, L2A, Stallion, Llama) existed only in dash.js. Their performance under LL-HLS (hls.js, HTTP/2 chunked delivery) was unknown. LL-HLS generates 6× more requests than dash.js, requiring new throughput measurement and playback-speed modules.

### Proposed Solution

Ported all 4 ABR schemes into hls.js v1.0.0 (~2,350 lines new/modified code).

L2A — online convex optimisation; Llama — harmonic mean for up-switches; Stallion — sliding-window mean±std; LoL+ — SOM-based classification with 5 modules.

Added sliding-window and harmonic-mean bandwidth estimators.

Evaluated on 3 LTE network traces: BelgiumCAR, NorwayMETRO, NyuBUS (50 ms delay, 0.08 % loss).

### Attained Results

LoL+ best overall: +49.8 % avg bitrate vs. other schemes at 3 s latency target.

LL-HLS ≈ LL-DASH in avg bitrate and latency — cross-protocol portability confirmed.

hls.js produces 6× more HTTP requests — ABR must account for sub-segment granularity.

Llama never up-shifted to the highest tier under any tested LL-HLS condition.

## 1.3  DASH Fast Start using HTTP/2

Cherif et al. ACM NOSSDAV 2015. http://dx.doi.org/10.1145/2736084.2736088

### Problem Statement

Sequential HTTP/1.1 DASH requests cause start-up delays that scale with RTT (e.g., 14 s at RTT=300 ms). When pushed segments are cached, bandwidth estimation from download time yields 4× over-estimates, causing the player to request an unsupportable quality tier.

### Proposed Solution

HTTP/2 server push — server proactively pushes MPD, init, and first N segments on MPD request.

WebSocket-over-HTTP/2 timing — "start/end sending" messages allow accurate bandwidth estimation for pushed content.

Client-Proxy (Node.js) — caches pushed segments, translates HTTP/1.1 from DASH-JS to HTTP/2.

Adaptive push — server stops pushing when buffer is full; resumes when buffer drains.

### Attained Results

Start-up time reduced by up to 50 % at RTT=300 ms (14 s → ~7 s).

WS-over-HTTP/2 RMSE < 1 Mbps vs. 2–11 Mbps for WS over dedicated TCP.

Nominal bitrate reached at segment 3–4 vs. segment 7 with traditional DASH.

Server push warms up the TCP congestion window faster, eliminating inter-segment idle time.

# 2. Comparison: Published Research vs. Assignment 2

Assignment 2 is a browser-based DASH client (vanilla JS + MSE API) streaming 2D video at five quality tiers (144p–1080p) with asymmetric EMA bandwidth estimation and three ABR modes: throughput-based, BOLA (buffer log-scale), and hybrid.

| Dimension | Assignment 2 | Published Research |
| --- | --- | --- |
| Media type | 2D H.264/H.265 fMP4 | VV-DASH: 3D point-cloud (Draco, V-PCC). Others: 2D CMAF/H.264. |
| Bandwidth estimation | Asymmetric EMA (α_down=0.5, α_up=0.15) | EMA, harmonic mean (Llama), sliding-window ±std (Stallion), SOM (LoL+), WS timing (HTTP/2 paper). |
| ABR algorithm | Throughput, BOLA, hybrid | LoL+, L2A, Llama, Stallion, server-side WS adaptation, bandwidth-based (VV-DASH). |
| Segment format | Standard ffmpeg fMP4 chunks + separate init.mp4 | DVV custom binary container (VV-DASH); CMAF/fMP4 (LL & HTTP/2 papers). |
| Buffer management | Low=4 s, High=8 s; deadline AbortController | Buffer-level controller (LL paper); push stops at threshold (HTTP/2); per-decoder buffer (VV-DASH). |
| Transport protocol | HTTP/1.1 / file:// | HTTP/2 + server push + WebSocket (Paper 3); CMAF CTE / HTTP/2 parts (Paper 2). |
| QoE metrics | Quality switches, stalls, avg bitrate, buffer level | Avg BR, RC, RD, SC, PS, LL (Paper 2); stalls + decoding rate (VV-DASH). |
| Stall handling | Abort + EMA reset to tier-0 × 0.5 | Playback-speed slowdown (LoL+); parallel decoding (VV-DASH); fast bitrate convergence (HTTP/2). |
| Live streaming | VoD only | LL paper: live, glass-to-glass latency 1.5–3 s. VV-DASH & HTTP/2: VoD. |
| Codec scope | avc1 only (codecs string from MPD) | Codec-agnostic DVV wrapper (VV-DASH); CMAF (others). |

# 3. Five Technical Gaps Remaining in Assignment 2

## Gap 1 — No QoE-Aware or Learning-Based ABR

Assignment 2 uses EMA × 0.75 and a simplified BOLA. Published ABRs (LoL+ SOM, L2A online convex optimisation) jointly optimise bitrate, stall risk, and latency without offline training, outperforming threshold policies by 49.8 % on LTE traces.

## Gap 2 — Bandwidth Estimator Ignores Timing Overhead

The EMA measures wall-clock fetch time, including TCP handshake and browser scheduling noise. Stallion adds a ±σ term to handle measurement variance; the HTTP/2 paper shows cached-push estimation can be 4× too high. The fixed α values have no variance correction.

## Gap 3 — HTTP/1.1 Only; No Multiplexing or Server Push

Each init and media chunk requires a separate TCP round trip. HTTP/2 server push cut start-up time by 50 % at RTT=300 ms (Paper 3). The assignment also has no live-streaming support, unlike the LL-DASH/HLS paper.

## Gap 4 — No Segment Batching (DVV-Equivalent)

Chunks are fetched sequentially one at a time. VV-DASH's DVV batching gained 13.2 % throughput and 51 % faster decoding. Audio and video are also fetched in separate requests with no header coordination.

## Gap 5 — No Playback-Speed Control

On buffer drain, the only action is abort + EMA reset. LoL+'s speed-control module adjusts playback rate by ±5–10 % to buy download time without a quality drop — essential for any live or near-live deployment.

# 4. What Must Be Done to Raise the Deliverables

The following improvements are ordered by priority:

Variance-aware bandwidth estimator. Replace fixed-α EMA with a sliding window tracking mean ± σ (last N segments). Gate up-switches only when mean − σ > next_tier.bitrate.

QoE objective function. Define Q = w1·BR − w2·switches − w3·RD − w4·LL and drive ABR by maximising Q, logging the score alongside per-chunk telemetry.

HTTP/2 Node.js proxy. Add a proxy that speaks HTTP/2 to the server and HTTP/1.1 to the MSE player; enable server push of the first 3–5 segments after the MPD response.

Playback-speed control. Adjust video.playbackRate within [0.95, 1.05] based on (bufferLevel − target) before resorting to abort, preventing unnecessary quality drops.

Multiplexed audio+video segment. Merge audio and video into one interleaved fMP4 per chunk to halve HTTP round trips and synchronise streams at the container level.

Reproducible evaluation under tc-netem. Emulate BelgiumCAR, NorwayMETRO, NyuBUS profiles; run 5 trials per ABR mode; report mean ± std of Avg BR, RD, RC, SC, LL.

Formal BOLA-BASIC reference. Implement the full utility function V·(p_m − γ_p·p_{m−1}) / (Q_max − Q(t) + 1) (Spiteri et al. 2016) and verify against published baselines.

# 5. Five Common Issues & How to Deal with Them

## Issue 1 — Bandwidth Estimation Inaccuracy (Bursts & Idle Periods)

Where it appears:

Assignment 2: EMA seeded by a cold-TCP first chunk (underestimate); stale after buffer-full idle gap.

Research: LoL+ has an idle-period correction module; Paper 3 shows push caching causes 4× overestimates.

How to deal with it:

(a) Discount EMA by 0.8× after idle > 2 s. (b) Sliding window with IQR outlier removal. (c) Use WebSocket timing for pushed segments.

## Issue 2 — Quality Oscillation / Bitrate Instability

Where it appears:

Assignment 2: EMA near a tier boundary causes per-chunk flipping; no hysteresis enforced.

Research: L2A oscillates on BelgiumCAR; Llama never up-shifts — both extremes hurt QoE.

How to deal with it:

(a) Require EMA > next_tier × 1.1 for two segments to switch up. (b) Enforce 3-segment minimum dwell at each quality level. (c) Add switch penalty w2 to the QoE objective.

## Issue 3 — Start-Up Delay & Cold-Start Bias

Where it appears:

Assignment 2: no bandwidth estimate at chunk_1 → defaults to 144p regardless of capacity.

Research: Paper 3 cuts start-up 50 % via server push; LoL+ avoids slow ramp-up at 1.5 s target.

How to deal with it:

(a) Send a 64 KB range-request probe before the main loop to seed the EMA. (b) Default first segment to 480p; downgrade on segment 2 if probe is too low. (c) Use HTTP/2 push of first 3 segments.

## Issue 4 — Stall Recovery is Too Aggressive

Where it appears:

Assignment 2: abort + reset EMA to tier-0 × 0.5 (≈0.04 Mbps) forces 144p even on transient spikes; slow multi-step recovery follows.

Research: VV-DASH recovers gracefully over 9 segments; LoL+ uses speed slowdown to avoid stalls entirely.

How to deal with it:

(a) Replace reset with EMA × 0.5 (not floor). (b) First slow playback to 0.9× before aborting. (c) Ramp back in two quality steps with 3-segment dwell each.

## Issue 5 — Lack of Reproducible Evaluation

Where it appears:

Assignment 2: manual browser observation, no scripted emulation, no repeated trials; results vary with browser scheduler load.

Research: all three papers use tc-netem or HTB; 5–10 trials per configuration; results reported as mean ± std.

How to deal with it:

(a) Puppeteer test harness + tc-netem for deterministic bandwidth shaping. (b) Standard profiles (BelgiumCAR, NorwayMETRO, NyuBUS); 5 trials each. (c) Version-lock deps and document OS/kernel for full reproducibility.

# References

[1] Heidarirad & Wang. VV-DASH. ACM MMSys 2025. https://doi.org/10.1145/3712676.3718339

[2] Bentaleb et al. Low Latency Live Streaming in DASH and HLS. ACM MM 2022. https://doi.org/10.1145/3503161.3548544

[3] Cherif et al. DASH Fast Start using HTTP/2. ACM NOSSDAV 2015. http://dx.doi.org/10.1145/2736084.2736088

[4] Spiteri et al. BOLA. IEEE INFOCOM 2016. https://doi.org/10.1109/INFOCOM.2016.7524428

[5] Ansari & Wang. iStream Player. ACM NOSSDAV 2023. https://doi.org/10.1145/3592473.3592569